ê 443/2009 (adapted)

ð new

ANNEX I

PART A. SPECIFIC EMISSIONS TARGETS Ö FOR PASSENGER CARS Õ

1.	ð For the calendar year 2020 ï T the specific  emissions of CO2 for each new passenger car, measured in grams per kilometre, shall , for the purposes of the calculations in this Annex, be determined in accordance with the following formulae:

(a)	From 2012 to 2015:

Specific emissions of CO2 = 130 + a × (M – M0)

Where:

| M | = | mass of the vehicle in kilograms (kg) |
| --- | --- | --- |
| M0 | = | 1372,0 |
| a | = | 0,0457 |

ê 6/2015 Art.1 (adapted)

(b)	From 2016:

Specific emission of CO2 = 130 + a × (M – M0)

Where:

| M | = | mass of the vehicle in kilograms (kg) |
| --- | --- | --- |
| M0 | = | 1 392,4 |
| a | = | 0,0457 |

ê 333/2014 Art. 1.13 (adapted)

ð new

From 2020:

Specific emission of CO2 = 95 + a ·× (M – M0)

Where:

| M | = | Mass ð in running order ï of the vehicle in kilograms (kg) |
| --- | --- | --- |
| M0 | = | The value adopted pursuant to Article 13(2) ð 1 379.88 ï |
| a | = | 0.,0333 |

ê 443/2009 (adapted)

2.	The specific emissions target for a manufacturer in a calendar year Ö 2020 Õ shall be calculated as the average of the specific emissions of CO2 of each new passenger car registered in that calendar year of which it is the manufacturer.

ê 2017/1502 Art. 1 and Annex pt. 1 (adapted)

ð new

3.	The specific emission reference target for a manufacturer in 2021 shall be calculated as follows:

WLTP specific emission reference target = WLTPCO2 ·

Where:

WLTPCO2	is the average specific emissions of CO2 in 2020 determined in accordance with Annex XXI to Commission Regulation (EU) 2017/1151 and calculated in accordance with the sixth second  indent in the second paragraph of Article 4(3)  of this Regulation, without including CO2 savings resulting from the application of Articles 5a and 12 11  of this Regulation;

NEDCCO2 	is the average specific emissions of CO2 in 2020 determined in accordance with Commission Implementing Regulation (EU) 2017/1153 and calculated in accordance with the sixth Ö second Õ indent in the second paragraph of Article 4(3) of this Regulation, without including CO2 savings resulting from the application of Articles 5a and 12 11 of this Regulation

NEDC2020target	is the 2020 specific emissions target calculated in accordance with points 1(c) and 2 of this Annex.

4.	Starting from ð For the calendar years ï  2021 ð to 2024 ï, the specific emissions target for a manufacturer shall be calculated as follows:

Specific emissions target = WLTPreference target + a [(Mø-M0) – (Mø2020 – M0,2020)]

Where:

WLTPreference target 	is the 2021 Ö WLTP Õ specific emission reference target calculated in accordance with point 3;

a	is as defined in point 1(c) 0.0333;

Mø	is the average of the mass Ö in running order Õ (M) as defined in point 1 of the new registered vehicles in the target year in kilograms (kg);

M0	 is ð 1379.88 in 2021, and ï as defined in point 1 Ö Article 13(1)(a) Õ ð for the period 2022, 2023 and 2024 ï;

Mø2020	is the average of the mass Ö in running order Õ (M) as defined in point 1of the new registered vehicles in 2020 in kilograms (kg);

M0,2020 	is ð 1379.88 ï the M0 value applicable in the reference year 2020.

5.	For a manufacturer that has been granted a derogation with regard to a specific NEDC based emissions target in 2021, the WLTP based derogation target shall be calculated as follows:

Derogation target2021 = WLTPCO2 ·

Where:

WLTPCO2	is as defined in point 3;

NEDCCO2 	is as defined in point 3;

NEDC2021target	is the 2021 specific emissions target granted by the Commission pursuant to Article 11 10 of this Regulation.

ò new

6.	From 1 January 2025, the EU fleet-wide targets and the specific emissions targets of CO2 for a manufacturer shall be calculated as follows:

6.1.	EU fleet-wide targets for 2025 and 2030

6.1.1.	EU fleet-wide target for 2025 to 2029

EU fleet-wide target2025 = EU fleet-wide target2021 · (1 - reduction factor2025)

Where,

EU fleet-wide target2021	is the average, weighted on the number of newly registered cars of each individual manufacturer, of the specific emissions targets determined for each individual manufacturer in 2021 in accordance with point 4

Reduction factor2025	is the reduction specified in Article 1(4)(a)

6.1.2.	EU fleet-wide target for 2030 onwards

EU fleet-wide target2030 = EU fleet-wide target2021 · (1 - reduction factor2030)

Where,

EU fleet-wide target2021	is the average, weighted on the number of newly registered cars of each individual manufacturer, of the specific emissions targets determined for each individual manufacturer in 2021 in accordance with point 4

Reduction factor2030	is the reduction specified in Article 1(5)(a)

6.2.	Specific emissions reference targets from 2025 onwards

6.2.1.	2025 to 2029

The specific emissions reference target = EU fleet-wide target2025 + a2025 · (TM-TM0)

Where,

EU fleet-wide target2025	is as determined in accordance with point 6.1.1

a2025	is

where,

a2021 	is the slope of the best fitting straight line established by applying the linear least squares fitting method to the test mass (explanatory variable) and the specific CO2 emissions (dependent variable) of each individual vehicle in the 2021 EU fleet

average emissions2021	is the average of the specific emissions of CO2 of all newly registered vehicles in 2021 of those manufacturers for which a specific emissions target is calculated in accordance with point 4

TM	is the average test mass in kilograms of all newly registered vehicles of the manufacturer in the relevant calendar year

TM0		is the value determined in accordance with Article 13(1)(d)

6.2.2.	2030 onwards

The specific emissions reference target = EU fleet-wide target2030 + a2030 · (TM-TM0)

Where,

EU fleet-wide target2030	is as determined in accordance with point 6.1.2

a2030					is

where,

a2021 	is the slope of the best fitting straight line established by applying the linear least squares fitting method to the test mass  (explanatory variable) and the specific CO2 emissions (dependent variable) of each individual vehicle in the 2021 EU fleet

average emissions2021	is the average of the specific emissions of CO2 of all newly registered vehicles in 2021 of those manufacturers for which a specific emissions target is calculated in accordance with point 4

TM	is the average test mass in kilograms of all newly registered vehicles of the manufacturer in the relevant calendar year

TM0		is the value determined in accordance with Article 13(1)(d)

6.3.	The specific emissions target from 2025 onwards

Specific emissions target = specific emissions reference target · ZLEV factor

Where,

Specific emissions reference target	is the specific emissions reference target of CO2 determined in accordance with point 6.2.1 for the period 2025 to 2029 and 6.2.2 for 2030 onwards

ZLEV factor	is (1+y-x), unless this sum is larger than 1.05 or lower than 1.0 in which case the ZLEV factor shall be set to 1.05 or 1.0 as the case may be

Where,

y	is the share of zero- and low-emission vehicles in the manufacturer's fleet of newly registered passenger cars calculated as the total number of zero- and low-emission vehicles, where each of them is counted as ZLEVspecific in accordance with the formula below, divided by the total number of passenger cars registered in the relevant calendar year

x	is 15% in the years 2025 to 2029 and 30% in 2030 onwards.

ê 510/2011 (adapted)

ð new

ANNEX I

- PART B.  SPECIFIC CO2 EMISSIONS TARGETS Ö FOR LIGHT COMMERCIAL VEHICLES Õ

1.	 ð In 2020 ï  T the indicative specific emissions of CO2 for each light commercial vehicle, measured in grams per kilometre, shall be determined in accordance with the following formulae:

(a)	from 2014 to 2017:

Indicative specific emissions of CO2 = 175 + a × (M – M0)

where:

| M | = | mass of the vehicle in kilograms (kg) |
| --- | --- | --- |
| M0 | = | 1706,0 |
| a | = | 0,093; |

ê 748/2017 Art. 1 (adapted)

(b)	from 2018:

Specific emission of CO2 = 175 + a × (M – M0)

Where:

| M | = | mass of the vehicle in kilograms (kg) |
| --- | --- | --- |
| M0 | = | 1766,4 |
| a | = | 0,093; |

ê 253/2014 Art. 1.7 (adapted)

ð new

(c)	from 2020:

Indicative sSpecific emissions of CO2 = 147 + a · (M - M0)

where:

| M | = | mass ð in running order ï of the vehicle in kilograms (kg) |
| --- | --- | --- |
| M0 | = | the value adopted pursuant to Article 13(5) ð 1 766.4 ï |
| a | = | 0.,096. |

ê 510/2011

ð new

2.	The specific emissions target for a manufacturer in a calendar year ð 2020 ï shall be calculated as the average of the indicative specific emissions of CO2 of each new light commercial vehicle registered in that calendar year of which it is the manufacturer.

ê 2017/1499 Art. 1 Annex pt.1 (adapted)

ð new

3.	The specific emission reference target for a manufacturer in 2021 shall be calculated as follows:

WLTP specific emission reference target =Where:

WLTPCO2	is the average specific emissions of CO2 in 2020 determined in accordance with Annex XXI to Commission Regulation (EU) 2017/1151 without including CO2 savings resulting from the application of Article 12 11  of this Regulation;

NEDCCO2 	is the average specific emissions of CO2 in 2020 determined in accordance with Implementing Regulation (EU) 2017/1152, without including CO2 savings resulting from the application of Article 12 11 of this Regulation;

NEDC2020target	is the 2020 specific emissions target calculated in accordance with point 1(c) and 2 of this Annex.

4.	Starting from ð For the calendar years ï 2021 ð to 2024 ï, the specific emissions target for a manufacturer shall be calculated as follows:

Specific emissions target= WLTPreference target + a [(Mø-M0) – (Mø2020 – M0,2020)]

Where:

WLTPreference target	is the 2021 Ö WLTP Õ specific emission reference target calculated in accordance with point 3;

a	is 0.096 a as defined in point 1(c);

Mø	is the average of the mass ð in running order ï (M) as defined in point 1 of the new registered light commercial vehicles in the relevant target year in kilograms (kg);

M0	is M0 as defined in point 1(c) ð 1 766.4 in 2020 and, for the period 2021, 2022 and 2023, the value adopted pursuant to Article 13(5) of Regulation (EU) No 510/2011, and for 2024 the value adopted pursuant to Article 13(1)(b) of this Regulationï ;

Mø2020	is the average of the mass ð in running order ï (M) as defined in point 1 of the new registered light commercial vehicles in 2020 in kilograms (kg);

M0,2020	is ð 1 766.4 ï the M0 value applicable in the reference year 2020.

5.	For a manufacturer that has been granted a derogation with regard to a specific NEDC based emissions target in 2021, the WLTP based derogation target shall be calculated as follows:

Derogation target2021 = WLTPCO2 ·

Where:

WLTPCO2	is WLTPCO2 as defined in point 3;

NEDCCO2 	is NEDCCO2 as defined in point 3;

NEDC2021target	is the 2021 specific emissions target granted by the Commission pursuant to Article 1011 of this Regulation.

ò new

6.	From 1 January 2025, the EU fleet-wide targets and the specific emissions target of CO2 for a manufacturer shall be calculated as follows:

6.1.	The EU fleet-wide targets for 2025 and 2030

6.1.1.	EU fleet-wide target for 2025 to 2029

EU fleet-wide target2025 = EU fleet-wide target2021 · (1 - reduction factor2025)

Where,

EU fleet-wide target2021	is the average, weighted on the number of newly registered light commercial vehicles of each individual manufacturer, of the specific emissions targets determined for each individual manufacturer in 2021 in accordance with point 4

Reduction factor2025	is the reduction specified in Article 1(4)(b)

6.1.2.	EU fleet-wide target for 2030 onwards

EU fleet-wide target2030 = EU fleet-wide target2021 · (1 - reduction factor2030)

Where,

EU fleet-wide target2021	is the average, weighted on the number of newly registered light commercial vehicles of each individual manufacturer, of the specific emissions targets determined for each individual manufacturer in 2021 in accordance with point 4

Reduction factor2030	is the reduction specified in Article 1(5)(b)

6.2.	The specific emissions reference target from 2025 onwards

6.2.1.	2025 to 2029

The specific emissions reference target = EU fleet-wide target2025 + α · (TM-TM0)

Where,

EU fleet-wide target2025	is as determined in accordance with point 6.1.1

α	is a2025 where the average test mass of a manufacturer's newly registered vehicles is equal to or lower than TM0 determined in accordance with Article 13(1)(d) and a2021 where the average test mass of a manufacturer's newly registered vehicles is higher than TM0 determined in accordance with Article 13(1)(d),

where,

a2025	is

a2021 	is the slope of the best fitting straight line established by applying the linear least squares fitting method to the test mass (explanatory variable) and the specific CO2 emissions (dependent variable) of each newly registered vehicle in the  2021 EU fleet

average emissions2021	is the average of the specific emissions of CO2 of all newly registered vehicles in 2021 of those manufacturers for which a specific emissions target is calculated in accordance with point 4

TM	is the average test mass in kilograms of all newly registered vehicles of the manufacturer in the relevant calendar year

TM0	is the value determined in accordance with Article 13(1)(d)

6.2.2.	2030 onwards

The specific emissions reference target = EU fleet-wide target2030 + α · (TM-TM0)

Where,

EU fleet-wide target2030	is as determined in accordance with point 6.1.2

α	is a2030 where the average test mass of a manufacturer's newly registered vehicles is equal to or lower than TM0 determined in accordance with Article 13(1)(d) and a2021 where the average test mass of a manufacturer's newly registered vehicles is higher than TM0 determined in accordance with Article 13(1)(d),

where,

a2030	is

a2021 	is the slope of the best fitting straight line established by applying the linear least squares fitting method to the test mass (explanatory variable) and the specific CO2 emissions (dependent variable) of each newly registered vehicle in the 2021 EU fleet

average emissions2021	is the average of the specific emissions of CO2 of all newly registered vehicles in 2021 of those manufacturers for which a specific emissions target is calculated in accordance with point 4

TM	is the average test mass in kilograms of all newly registered vehicles of the manufacturer in the relevant calendar year

TM0	is the value determined in accordance with Article 13(1)(d)

6.3. Specific emissions targets from 2025 onwards

6.3.1. From 2025 to 2029

The specific emissions target = (specific emissions reference target – (øtargets – EU fleet-wide target2025)) · ZLEV factor

Where,

Specific emissions reference target	is the specific emissions reference target for the manufacturer determined in accordance with point 6.2.1

øtargets	is the average, weighted on the number of newly registered light commercial vehicles of each individual manufacturer, of all the specific emissions reference targets determined in accordance with point 6.2.1

ZLEV factor	is (1+y-x), unless this sum is larger than 1.05 or lower than 1.0 in which case the ZLEV factor shall be set to 1.05 or 1.0 as the case may be

Where,

y	is the share of zero- and low-emission vehicles in the manufacturer's fleet of newly registered light commercial vehicles calculated as the total number of zero- and low-emission vehicles, where each of them is counted as ZLEVspecific in accordance with the formula below, divided by the total number of light commercial vehicles registered in the relevant calendar year

x	is 15%

6.3.2. From 2030 onwards

The specific emissions target = (specific emissions reference target – (øtargets – EU fleet-wide target2030)) · ZLEV factor

Where,

Specific emissions reference target	is the specific emissions reference target for the manufacturer determined in accordance with point 6.2.2

øtargets	is the average, weighted on the number of newly registered light commercial vehicles of each individual manufacturer, of all the specific emissions reference targets determined in accordance with point 6.2.2

ZLEV factor	is (1+y-x), unless this sum is larger than 1.05 or lower than 1.0 in which case the ZLEV factor shall be set to 1.05 or 1.0 as the case may be

Where,

y	is the share of zero- and low-emission vehicles in the manufacturer's fleet of newly registered light commercial vehicles calculated as the total number of zero- and low-emission vehicles, where each of them is counted as ZLEVspecific in accordance with the formula below, divided by the total number of light commercial vehicles registered in the relevant calendar year

x	is 30%

ê 397/2013 Art.1 and Annex  (adapted)

ANNEX II

- MONITORING AND REPORTING EMISSIONS Ö FROM NEW PASSENGER CARS Õ

- PART A —	Collection of data on new passenger cars and determination of CO2 monitoring information

ê 2017/1502 Art. 1 and Annex pt. 2(a)

ð new

1.	Member States shall, for each calendar year, record the following detailed data for each new passenger car registered ð as an M1 vehicle ï in their territory:

(a)	the manufacturer;

(b)	the type-approval number with its extension;

(c)	the type, variant, and version (where applicable);

(d)	make and commercial name;

(e)	category of vehicle type-approved;

(f)	total number of new registrations;

(g)	mass in running order;

(h)	the specific emissions of CO2 (NEDC and WLTP);

(i)	footprint: the wheel base, the track width steering axle and the track width other axle;

(j)	the fuel type and fuel mode;

(k)	engine capacity;

(l)	electric energy consumption;

(m)	code for the innovative technology or group of innovative technologies and the CO2 emissions reduction due to that technology (NEDC and WLTP);

(n)	maximum net power;

(o)	vehicle identification number;

(p)	WLTP test mass;

(q)	deviation and verification factors referred to in point 3.2.8 of Annex I to Implementing Regulation (EU) 2017/1153;

(r)	category of vehicle registered;.

ò new

(s)	vehicle family identification number;

(t)	 electric range, where applicable.

ê 2017/1502 Art. 1 and Annex pt. 2(a) (adapted)

However, for the calendar year 2017, the data referred to in point (g) as regards WLTP CO2 emissions values, and in point (l), as regards WLTP eco-innovation savings, as well as the data referred to in points (n), (o) and (q) may be reported on a voluntary basis.

Starting from calendar year 2018, Member States shall make available to the Commission, in accordance with Article 87 all parameters listed in this point as specified in the format in Section 2 of Part CB.

Member States shall make available the data referred to in point (f) for calendar years 2017 and 2018.

ê 397/2013 Art. 1 and Annex  (adapted)

2.	The detailed data referred to in point 1 shall be taken from the certificate of conformity of the relevant passenger car. or be consistent with the certificate of conformity issued by the manufacturer of the relevant passenger car. Where the certificate of conformity is not used, Member States shall put the necessary measures in place to ensure adequate accuracy in the monitoring procedure. Where both a minimum value and a maximum value are specified for the mass or footprint values referred to in point 1(i) for a passenger car, Member States shall use only the maximum figure for the purposes of this Regulation. In the case of bi-fuelled vehicles (petrol/gas), the certificates of conformity of which bear specific CO2 emissions figures for both types of fuel, Member States shall use only the figure measured for gas.

3.	Member States shall, for each calendar year, determine:

(a)	the sources used for the collection of the detailed data referred to in point 1;

(ba)	the total number of new registrations of new passenger cars subject to EC type-approval;

(cb)	the total number of new registrations of new individually approved passenger cars;

(dc)	the total number of new registrations of new passenger cars approved nationally in small series.;

(e)	the percentage of all fuel filling stations on their territory providing E85.

- PART B —	Methodology for determining CO2 monitoring information for new passenger cars

Monitoring information which Member States are required to determine in accordance with points 1 and 3 of Part A shall be determined in accordance with the methodology in this Part.

1.	Number of new passenger cars registered

Member States shall determine the number of new passenger cars registered within their territory in the respective monitoring year divided into vehicles subject to EC type-approval, individual approvals and national approvals of small series.

2.	The distribution by version of new passenger cars

For each version of each variant of each type of new passenger car, the number of newly registered passenger cars and the detailed data referred to in point 1 of Part A shall be recorded.

3.	The fuel stations in their territory that supply E85 fuel shall be indicated in accordance with Article 6 of Commission Regulation (EU) No 1014/2010.

ê 2017/1502 Art. 1 and Annex pt. 2(b) (adapted)

ð new

- PART BC  —	Format for the transmission of data

For each year, Member States shall report the information specified in points 1 and 3 of Part A in the following formats:

Section 1 - Aggregated monitoring data

| Member State |  |
| --- | --- |
| Year |  |
| Data source |  |
| Total number of new registrations of new passenger cars subject to EC type-approval |  |
| Total number of new registrations of new individually approved passenger cars |  |
| Total number of new registrations of new passenger cars approved nationally in small series |  |

Section 2 – Detailed monitoring data – one vehicle record

| Reference to Point 1 of Part A | Detailed data per vehicle registered |
| --- | --- |
| (a) | Manufacturer name EU standard denomination |
|  | Manufacturer name OEM declaration |
|  | Manufacturer name in Member State registry1 |
| (b) | Type approval number and its extension |
| (c) | Type |
|  | Variant |
|  | Version |
| (d) | Make and commercial name |
| (e) | Category of vehicle type approved |
| (f) | Total number of new registrations (for 2017 and 2018) |
| (g) | Mass in running order |
| (h) | Specific CO2 emissions (combined) NEDC value ð until 31 December 2020 except for vehicles that fall within the scope of Article 5 for which the NEDC value shall be determined until 31 December 2022 in accordance with Article 5 of Implementing Regulation (EU) 2017/1153 ï |
|  | Specific CO2 emissions (combined) WLTP value (from 2019) |
| (i) | Wheel base |
|  | Track width steering axle (Axle 1) |
|  | Track width other axle (Axle 2) |
| (j) | Fuel type |
|  | Fuel mode |
| (k) | Engine capacity (cm3) |
| (l) | Electric energy consumption (Wh/km) |
| (m) | Code of the eco-innovation(s) |
|  | Total NEDC CO2 emissions savings due to the eco-innovation(s) ð until 2020 inclusive ï |
|  | Total WLTP CO2 emissions savings due to the eco-innovation(s) (from 2019) |
| (n) | Maximum net power |
| (o) | Vehicle identification number (from 2019) |
| (p) | WLTP test mass (from 2019) |
| (q) | Deviation factor De (where available) |
|  | Verification factor (where available) |
| (r) | Category of vehicle registered |
| (s) | ð Vehicle family identification number ï |
| (t) | ð Electric range, where applicable ï |

Ö Notes: Õ

1	In the case of the national small series approvals (NSS) or the individual approvals (IVA), the manufacturer name shall be provided in the column "Manufacturer name in Member State registry" whilst in the column "Manufacturer name EU standard denomination" either of the following shall be indicated: "AA-NSS" or "AA-IVA" as the case may be.

ê 510/2011 (adapted)

Annex IIIII

- MONITORING AND REPORTING OF EMISSIONS Ö FROM LIGHT COMMERCIAL VEHICLES Õ

- A.	Collection of data on light commercial vehicles and determination of CO2 monitoring information

ê 404/2014 Art. 1 and Annex pt.1(a)

è1 2017/1499 Art. 1 and Annex pt.2(a)(i)

è2 2017/1499

ð new

- 1.	Detailed data

- 1.1.	Complete vehicles registered as N1

In the case of EC type-approved complete vehicles registered as N1, Member States shall, for each calendar year, record the following detailed data for each new light commercial vehicle the first time that it is registered in their territory:

(a)	the manufacturer;

(b)	the type-approval number with its extension;

(c)	the type, variant, and version;

(d)	make;

(e)	category of vehicle type-approved;

(f)	category of vehicle registered;

(g)	the specific emissions of CO2 è1 (NEDC and WLTP) ç  ;

(h)	mass in running order;

(i)	technically permissible maximum laden mass;

(j)	footprint: the wheel base, the track width steering axle and the track width other axle;

(k)	the fuel type and fuel mode;

(l)	engine capacity;

(m)	electric energy consumption;

(n)	code of the innovative technology or group of innovative technologies and the CO2 emissions reduction due to that technology è2 (NEDC and WLTP) ç ;

(o)	the vehicle identification number;.

ê 2017/1499 Art.1 and Annex pt. 2(a)(i)

(p)	WLTP test mass;

(q)	Deviation and verification factors referred to in point 3.2.8 of Annex I to Implementing Regulation (EU) 2017/1152;

(r)	Vehicle family identification number determined in accordance with point 5.0 of Annex XXI to Regulation (EU) 2017/1151;.

ò new

(s)	electric range, where applicable.

ê 404/2014 Art. 1 and Annex pt. 1(a)

The format set out in Section 2 of Part C shall be used.

ê 2017/1499 Art. 1 and Annex pt. 2(a)(ii) (adapted)

For the calendar year 2017, the data referred to in point (g), as regards WLTP CO2 emissions values, and in point (n), as regards WLTP eco-innovation savings, as well as the data referred to in points (p) and (r) may be reported on a voluntary basis.

Starting from calendar year 2018, Member States shall make available to the Commission, in accordance with Article 87, all parameters listed in this point as specified in the format of Section 2 of Part C of this Annex.

ê 404/2014 Art. 1 and Annex pt. 1(a)

- 1.2.	Vehicles approved in a multi-stage process and registered as N1 vehicles

In the case of multi-stage vehicles registered as N1 vehicles, Member States shall, for each calendar year, record the following detailed data with regard to:

(a)	the base (incomplete) vehicle: the data specified in points (a), (b), (c), (d), (e), (g), (h), (i), (n) and (o) of point 1.1, or, instead of the data specified in (h) and (i), the default added mass provided as part of the type-approval information specified in point 2.17.2 of Annex I to Directive 2007/46/EC;

(b)	the base (complete) vehicle: the data specified in points (a), (b), (c), (d), (e), (g), (h), (i), (n) and (o) of point 1.1;

(c)	the completed vehicle: the data specified in points (a), (f), (g), (h), (j), (k), (l), (m) and (o) specified in point 1.1.

Where any of the data referred to in points (a) and (b) of this point cannot be provided for the base vehicle, the Member State shall provide data with regard to the completed vehicle instead.

The format set out in Section 2 of Part C shall be used for completed N1 vehicles.

The vehicle identification number referred to in point (o) of point 1.1 shall not be made public.

ê 510/2011 (adapted)

è1 205/2012 Art. 1 and Annex pt.1(a)

2.	 è1 The details referred to in point 1 shall be taken from the certificate of conformity or be consistent with the certificate of conformity issued by the manufacturer of the relevant light commercial vehicle. Where the certificate of conformity is not used, Member States shall put the necessary measures in place to ensure adequate accuracy in the monitoring procedure. ç  Where the certificate of conformity specifies both a minimum and a maximum mass for a light commercial vehicle, the Member States shall use only the maximum figure for the purpose of this Regulation. In the case of bi-fuelled vehicles (petrol/gas) the certificates of conformity of which bear specific CO2 emission figures for both types of fuel, Member States shall use only the figure measured for gas.

ê 404/2014 Art. 1 and Annex pt. 1(b)

3.	Member States shall, for each calendar year, determine:

(a)	the sources used for the collection of the detailed data referred to in point 1;

(ba)	the total number of new registrations of new light commercial vehicles subject to EC type-approval;

(cb)	the total number of new registrations of new light commercial vehicles subject to multi-stage type-approval, where available;

(dc)	the total number of new registrations of new individually approved light commercial vehicles;

(ed)	the total number of new registrations of new light commercial vehicles approved nationally in small series.

ê 404/2014 Art. 1 and Annex pt. 2(a)

- B.	Methodology for determining CO2 monitoring information for new light commercial vehicles

Monitoring information which Member States are required to determine in accordance with points 1 and 3 of Part A of this Annex shall be determined in accordance with the methodology in this Part.

- 1.	Number of new light commercial vehicles registered

Member States shall determine the number of new light commercial vehicles registered within their territory in the respective monitoring year divided into vehicles subject to EC type-approval, individual approvals and national approvals of small series and, where available, the number of multi-stage vehicles.

ê 510/2011 (adapted)

ð new

- 72.	Completed vehicles

In the case of multi-stage vehicles, the specific emissions of CO2 of completed vehicles shall be allocated to the manufacturer of the base vehicle.

In order to ensure that the values of CO2 emissions, fuel efficiency and mass of completed vehicles are representative, without placing an excessive burden on the manufacturer of the base vehicle, the Commission shall come forward with a specific monitoring procedure and shall ð where appropriate ï review and make the necessary amendments to the relevant type-approval legislation by 31 December 2011 at the latest.

When defining such a procedure, the Commission shall, if appropriate, determine how the mass and CO2 values are monitored, based on a table of CO2 values corresponding to different final inertia weight classes or based on only one CO2 value derived from the base vehicle mass plus a default added mass differentiated by N1 class. In the latter case, this mass would also be taken for Part C of this Annex.

The Commission shall also ensure that the manufacturer of the base vehicle has timely access to the mass and to the specific emissions of CO2 of the completed vehicle.

ê 404/2014 Art. 1 and Annex pt.2(c)

ð new

Notwithstanding that ð for the purpose of the calculation of the 2020 target in accordance with point 2 of Part B of Annex I ï the default added mass shall be taken fromfor Part C of this Annex, where that mass value cannot be determined, the mass in running order of the completed vehicle may be used for the provisional calculation of the specific emissions target referred to in Article 87 (4).

Where the base vehicle is a complete vehicle, the mass in running order of that vehicle shall be used for the calculation of the specific emissions target. However, where that mass value cannot be determined, the mass in running order of the completed vehicle may be used for the provisional calculation of the specific emissions target.

ê 404/2014 Art. 1 and Annex pt.3 (adapted)

è1 2017/1499 Art. 1 and Annex pt. 2(b)(i)

è2 2017/1499

ð new

- C.	Formats for transmission of data

For each year, Member States shall report the information specified in points 1 and 3 of Part A in the following format:

| Section 1 — Aggregated monitoring data |  |
| --- | --- |
| Member State |  |
| Year |  |
| Data source |  |
| Total number of new registrations of new light commercial vehicles subject to EC type-approval |  |
| Total number of new registrations of individually approved new light commercial vehicles |  |
| Total number of new registrations of new light commercial vehicles approved as national small series |  |
| Total number of new registrations of new light commercial vehicles subject to multi-stage type-approval (where available) |  |

| Section 2 — Detailed monitoring data — one vehicle record |  |
| --- | --- |
| Reference to Section 1.1 of Part A | Detailed data per vehicle registered(1) |
| (a) | Manufacturer name EU standard denomination(2) |
|  | Manufacturer name OEM declaration COMPLETE VEHICLE/BASE VEHICLE(3) |
|  | Manufacturer name OEM declaration COMPLETED VEHICLE(3) |
|  | Manufacturer name in Member State registry(2) |
| (b) | Type-approval number and its extension |
| (c) | Type |
|  | Variant |
|  | Version |
| (d) | Make |
| (e) | Category of vehicle type-approved |
| (f) | Category of vehicle registered |
| è1 (g) ç | è1 Specific CO2 emissions (combined) NEDC value ç ð until 31 December 2020 ï |
|  | è1 Specific CO2 emissions (combined) WLTP value (from 2018) ç |
| (h) | Mass in running order BASE VEHICLE |
|  | Mass in running order COMPLETED VEHICLE/COMPLETE VEHICLE |
| (i)(4) | Technically permissible maximum laden mass |
| (j) | Wheel base |
|  | Axle width steering axle (Axle 1) |
|  | Axle width other axle (Axle 2) |
| (k) | Fuel type |
|  | Fuel mode |
| (l) | Engine capacity (cm3) |
| (m) | Electric energy consumption (Wh/km) |
| è2 (n) ç | è2 Code of the eco-innovation(s) ç |
|  | è2 Total NEDC CO2 emissions savings due to the eco-innovation(s) ç ð until 31 December 2020 ï |
|  | è2 Total WLTP CO2 emissions savings due to the eco-innovation(s) (from 2018) ç |
| (o) | Vehicle identification number |

ê 2017/1499 Art. 1 and Annex pt. 2(b)(iii)

| (p) | WLTP test mass |
| --- | --- |
| (q) | Deviation factor De (where available) |
|  | Verification factor (where available) |
| (r) | Vehicle family identification number |

ò new

| (s) | electric range, where available |
| --- | --- |

ê 404/2014 Art.1 and Annex pt.3 (adapted)

| Point 2.17.2 of Annex I to Directive 2007/46/EC | Default added mass (where applicable in the case of multi-stage vehicles) |
| --- | --- |

Ö Notes: Õ

( 1 ) 	Where, in the case of multi-stage vehicles, data cannot be provided for the base vehicle, the Member State shall as a minimum provide the data specified in this format for the completed vehicle.

( 2 ) 	In the case of the national small series approvals (NSS) or the individual approvals (IVA), the manufacturer name shall be provided in the column ‘Manufacturer name in Member State registry’ whilst in the column ‘Manufacturer name EU standard denomination’ either of the following shall be indicated: ‘AA-NSS’ or ‘AA-IVA’ as the case may be.

( 3 ) 	In the case of multi-stage vehicles indicate the base (incomplete/complete) vehicle manufacturer. If the base vehicle manufacturer is not available indicate the manufacturer of the completed vehicle only.

( 4 ) 	In the case of multi-stage vehicles indicate the technically permissible maximum laden mass of the base vehicle.

( 5 ) 	In the case of multi-stage vehicles, the mass in running order and the technically permissible maximum laden mass of the base vehicle may be replaced by the default added mass specified in the type-approval information in accordance with point 2.17.2 of Annex I to Directive 2007/46/EC.

é

ANNEX IV

Repealed Regulations with lists of their successive amendments

| Regulation (EC) No 443/2009 of the European Parliament and of the Council |  | (OJ L 140, 5.6.2009, p. 1) |  |  |
| --- | --- | --- | --- | --- |
|  | Commission Regulation (EU) No 397/2013 |  | (OJ L 120, 1.5.2013, p. 4) |  |
|  | Regulation (EU) No 333/2014 of the European Parliament and of the Council |  | (OJ L 103, 5.4.2014, p. 15) |  |
|  | Commission Delegated Regulation (EU) 2015/6 Commission Delegated Regulation (EU) 2017/1502 |  | (OJ L 003, 7.1.2015, p. 1) (OJ L 221, 26.8.2017, p. 4) |  |
| Regulation (EU) No 510/2011 of the European Parliament and of the Council |  | (OJ L 145, 31.5.2011, p. 1) |  |  |
|  | Commission Delegated Regulation (EU) No 205/2012 |  | (OJ L 072, 10.3.2012, p. 2) |  |
|  | Regulation (EU) No 253/2014 of the European Parliament and of the Council |  | (OJ L 084, 20.3.2014, p. 38) |  |
|  | Commission Delegated Regulation (EU) No 404/2014 |  | (OJ L 121, 24.4.2014, p. 1) |  |
|  | Commission Delegated Regulation (EU) 2017/748 Commission Delegated Regulation (EU) 2017/1499 |  | (OJ L 113, 29.4.2017, p. 9) (OJ L 219, 25.8.2017, p. 1) |  |

_____________

ANNEX V

- Correlation Table

| Regulation (EC) No 443/2009 | Regulation (EU) No 510/2011 | This Regulation |
| --- | --- | --- |
| Article 1, first subparagraph | Article 1(1) | Article 1(1) |
| Article 1, second subparagraph | Article 1(2) | Article 1(2) |
| Article 1, third subparagraph | – | Article 1(3) |
| – | – | Article 1(4) |
| Article 2(1) | Article 2(1) | Article 2(1) |
| Article 2(2) | Article 2(2) | Article 2(2) |
| Article 2(3) | Article 2(3) | Article 2(3) |
| Article 2(4) | Article 2(4) | Article 2(4) |
| Article 3(1), introductory wording | Article 3(1), introductory wording | Article 3(1), introductory wording |
| Article 3(1), points a and b | Article 3(1), points a and b | Article 3(1), points a and b |
| – | Article 3(1), points c, d and e | Article 3(1), points c, d and e |
| Article 3(1), points c and d | Article 3(1), points f and g | Article 3(1), points f and g |
| Article 3(1), point e | Article 3(1), point j | – |
| Article 3(1), points f and g | Article 3(1), points h and i | Article 3(1), points h and i |
| – | – | Article 3(1), points j, k and l |
| Article 3(1), point k | – | – |
| Article 3(2) | Article 3(2) | Article 3(2) |
| Article 4(1) | – | – |
| – | – | Article 4(1), introductory wording |
| – | – | Article 4(1), point a |
| – | Article 4, first subparagraph | Article 4(1), point b |
| – | – | Article 4(1), point c |
| – | Article 4, second subparagraph | Article 4(2) |
| Article 4, second subparagraph | Article 4, third subparagraph | Article 4(3) |
| Article 5 | Article 5 | – |
| Article 5a | – | Article 5 |
| Article 6 | Article 6 | – |
| Article 7(1) | Article 7(1) | Article 6(1) |
| Article 7(2), points a, b and c | Article 7(2), points a, b and c | Article 6(2), points a, b and c |
| – | – | Article 6(2), point d |
| Article 7(3) | Article 7(3) | Article 6(3) |
| Article 7(4) | Article 7(4) | Article 6(4) |
| Article 7(5) | Article 7(5) | Article 6(5) |
| Article 7(6) | Article 7(6) | Article 6(6) |
| Article 7(7) | Article 7(7) | Article 6(7) |
| Article 8(1) | Article 8(1) | Article 7(1) |
| Article 8(2) | Article 8(2) | Article 7(2) |
| Article 8(3) | Article 8(3) | Article 7(3) |
| Article 8(4), first and second subparagraphs | Article 8(4), first and second subparagraphs | Article 7(4), first and second subparagraphs |
| Article 8(4), third subparagraph | – | Article 7(4), third subparagraph |
| Article 8(5), first subparagraph | Article 8(5) | Article 7(5) first subparagraph |
| Article 8(5), second subparagraph | Article 8(6) | – |
| Article 8(6) | Article 8(7) | – |
| Article 8(7) | – | Article 7(6), first subparagraph |
| – | – | Article 7(6), second subparagraph |
| – | Article 8(8) | – |
| Article 8(8) | – | – |
| Article 8(9) | Article 8(9) | Article 7(7) |
| – | – | Article 7(8) |
| – | Article 8(10) | Article 7(9) |
| Article 9(1) | Article 9(1) | Article 8(1) |
| Article 9(2), first subparagraph, introductory wording | Article 9(2), first subparagraph, introductory wording | Article 8(2) |
| Article 9(2), first subparagraph, point a | Article 9(2), first subparagraph, point a | – |
| Article 9(2), first subparagraph, point b | Article 9(2), first subparagraph, point b | Article 8(2) |
| Article 9(2), second subparagraph | Article 9(2), second subparagraph | Article 8(2), second subparagraph |
| Article 9(3) | Article 9(3) | Article 8(3) |
| Article 9(4) | Article 9(4) | Article 8(4) |
| Article 10(1), introductory wording | Article 10(1), introductory wording | Article 9(1), introductory wording |
| Article 10(1), points a, b, c, d and e | Article 10(1), points a, b, c, d and e | Article 9(1), points a, b, c, d and e |
| – | – | Article 9(1), point f |
| Article 10(2) | Article 10(2) | Article 9(2) |
| Article 11(1) | Article 11(1) | Article 10(1) |
| Article 11(2) | Article 11(2) | Article 10(2) |
| Article 11(3) | Article 11(3) | Article 10(3) |
| Article 11(4), first subparagraph | – | Article 10(4), first subparagraph |
| Article 11(4), second subparagraph, introductory wording | – | Article 10(4), second subparagraph, introductory wording |
| Article 11(4), second subparagraph, point a | – | Article 10(4), second subparagraph, point a |
| Article 11(4), second subparagraph, point b | – | – |
| Article 11(4), second subparagraph, point c | – | Article 10(4), second subparagraph, point b |
| Article 11(4), third and fourth subparagraphs | – | Article 10(4), third and fourth subparagraphs |
| Article 11(5) | Article 11(4) | Article 10(5) |
| Article 11(6) | Article 11(5) | Article 10(6) |
| Article 11(7) | Article 11(6) | Article 10(7) |
| Article 11(8) | Article 11(7) | Article 10(8) |
| Article 11(9) | Article 11(8) | Article 10(9) |
| Article 12(1), first subparagraph | Article 12(1), first subparagraph | Article 11(1), first subparagraph |
| Article 12(1), second subparagraph | – | Article 11(1), second subparagraph |
| Article 12(1), third subparagraph | Article 12(1), second subparagraph | Article 11(1), third subparagraph |
| – | – | Article 11(1), fourth subparagraph |
| Article 12(2) | Article 12(2) | Article 11(2) |
| Article 12(3) | Article 12(3) | Article 11(3) |
| Article 12(4) | Article 12(4) | Article 11(4) |
| – | – | Article 12(1) |
| – | – | Article 12(2) |
| – | – | Article 12(3) |
| Article 13(1) | – | – |
| – | – | Article 13, title |
| – | – | Article 13(1), first subparagraph, introductory wording |
| Article 13(2), first subparagraph | – | Article 13(1), point a |
| – | Article 13(2) | Article 13(1), points b |
|  |  | Article 13(1), points c and d |
| Article 13(2), second subparagraph | – | – |
| Article 13(2), third subparagraph | – | Article 13(2) |
| – | Article 13(1) | Article 14(1) |
| – | Article 13(4) | – |
| – | Article 13(5) | – |
| Article 13(3) | Article 13(6) | Article 14(2) |
| Article 13(4) | – | – |
| Article 13(5) | – | – |
| Article 13(6) | Article 13(3) | – |
| Article 13(7) | – | Article 14(3) |
| Article 14(1) | Article 14(1) | Article 15(1) |
| Article 14(2) | Article 14(2) | Article 15(2) |
| Article 14(3) | Article 14(3) | Article 15(3) |
| Article 14a(1) | Article 15(3) | Article 16(1) |
| Article 14a(2) | Article 15(1) | Article 16(2) |
| Article 14a(3) | Article 16 | Article 16(3) |
| Article 14a(4) | Article 15(2) | – |
| Article 14a(5) | Article 17 | – |
| – | – | Article 17 |
| Article 15 | – | Article 17 |
| Article 16 | Article 18 | Article 18 |
| Annex I | – | Annex I, part A |
| – | Annex I | Annex I, part B |
| Annex II, part A | – | Annex II, part A |
| Annex II, parts B | – | – |
| Annex II, part C | – | Annex II, part B |
| – | Annex II | Annex III |
| – | – | Annex IV |
| – | – | Annex V |

_____________

Commission Regulation (EU) 2017/1151 of 1 June 2017 supplementing Regulation (EC) No 715/2007 of the European Parliament and of the Council on type-approval of motor vehicles with respect to emissions from light passenger and commercial vehicles (Euro 5 and Euro 6) and on access to vehicle repair and maintenance information, amending Directive 2007/46/EC of the European Parliament and of the Council, Commission Regulation (EC) No 692/2008 and Commission Regulation (EU) No 1230/2012 and repealing Regulation (EC) No 692/2008 (OJ L 175, 7.7.2017, p. 1).

Commission Implementing Regulation (EU) 2017/1152 of 2 June 2017 setting out a methodology for determining the correlation parameters necessary for reflecting the change in the regulatory test procedure with regard to light commercial vehicles and amending Implementing Regulation (EU) No 293/2012 (OJ L 175, 7.7.2017, p. 644).’;

OJ L 293, 11.11.2010, p. 15.

ISO 3166 alpha-2 codes with the exception of Greece and the United Kingdom for which the codes are ‘EL’ and ‘UK’ respectively.

ISO 3166 alpha-2 codes with the exception of Greece and the United Kingdom for which the codes are ‘EL’ and ‘UK’ respectively.

In the case of multi-stage vehicles, the mass in running order and the technically permissible maximum laden mass of the base vehicle may be replaced by the default added mass specified in the type-approval information in accordance with point 2.17.2 of Annex I to Directive 2007/46/EC.