EN

|  | EUROPEAN COMMISSION |
| --- | --- |

Brussels, 10.11.2010

COM(2010) 655 final

REPORT FROM THE COMMISSION
TO THE EUROPEAN PARLIAMENT, THE COUNCIL AND THE EUROPEAN ECONOMIC AND SOCIAL COMMITTEE

Monitoring the CO2 emissions from new passenger cars in the EU:
data for 2009

REPORT FROM THE COMMISSION
TO THE EUROPEAN PARLIAMENT, THE COUNCIL AND THE EUROPEAN ECONOMIC AND SOCIAL COMMITTEE

Monitoring the CO2 emissions from new passenger cars in the EU:
data for 2009

1.	INTRODUCTION

According to Article 9 of Decision (EC) 1753/2000, the Commission shall submit annual reports based on the monitoring data submitted by Member States to the European Parliament and Council. The present Report concerns the monitoring data for 2009.

From 2010 monitoring and reporting under Regulation (EC) No 443/2009 replaces the reporting based on Decision (EC) 1753/2000 since that Decision is repealed by the Regulation.

2.	TRENDS IN NEW PASSENGER CARS

2.1.	Data quality and processing

All Member States have submitted data for new passenger car registrations for 2009. Manual re-processing of data was necessary for a number of Member States due to the recording of other fuel types than those agreed upon, the handling of unknown vehicles, the application of the correction factor and other data implausibilities. These issues were addressed during the data evaluation process and were successfully solved without significant data losses. Caution should be used when analyzing the time series as not all EU Member States are included for all years as it can be seen from the tables in section 3. While the highest effort is taken to present consistent series, some breaks in trends due to methodology and monitoring improvements might occur.

In comparison to the reports in previous years, the aggregated data in this report refer to all fuels (previously only figures with respect to petrol and diesel were reported) and in contrast to earlier reports, the CO2 emissions figures are not corrected by 0.7%. This correction of 0.7% had been applied in assessing the progress made by the manufacturing associations ACEA, JAMA and KAMA, in order to compensate for a change in the test procedure that took place after their voluntary commitments were made. These commitments have now been superseded by the targets specified in Regulation (EC) No 443/2009. For reasons of continuity with previous reports, data per vehicle manufacturer association continue to be presented.

Data for alternative fuel vehicles (AFV) are included for the second time in the monitoring report due to their increasing market share and improved data quality. This includes vehicles in the following fuel categories: Liquefied Petroleum Gas (LPG), Natural Gas (NG), electricity, hydrogen, dual fuel, petrol-bioethanol, petrol-LPG, petrol-NG as well as vehicles reported in the category Other.

2.2. Average CO2 emissions from new passenger cars

The average specific CO2 emissions of new passenger cars registered in the European Union in 2009 were 145.7 gCO2/km. This represents a decrease by 5.1%, or 7.9 gCO2/km, from the previous year (153.6 gCO2/km in 2008) which is the largest relative drop in specific emissions since the beginning of the monitoring scheme. Some of this reduction may be due to the financial and economic crisis as well as to the design of the scrappage schemes implemented in several Member States. The data also indicate that there has been some downsizing of the car fleet, as the average engine power, vehicle mass and engine capacity slightly decreased in 2009. Diesel powered vehicles improved by nearly 6 grams, petrol powered vehicles improved by almost 9 grams and AFV vehicles by slightly more than 11 grams in 2009. The difference among new diesel and petrol vehicles decreased to around 2.3 grams. It is worth noting that 10 years ago there was more than 17 grams difference between new gasoline and diesel vehicles.

The proportion of new petrol cars surpassed that of diesel vehicles in 2009. In 2009 the share of AFV increased significantly. This is mostly due to the increase of petrol-LPG vehicles registered in Italy, which itself accounts for 92.8% of all new petrol-LPG passenger cars registered in EU27.

Table 1: Average CO2 emissions from new passenger cars by fuel (EU27)

| gCO2/km | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| All fuels | 172.2 | 169.7 | 167.2 | 165.5 | 163.4 | 162.4 | 161.3 | 158.7 | 153.6 | 145.7 |
| Petrol | 177.4 | 175.3 | 173.5 | 171.7 | 170.0 | 168.1 | 164.9 | 161.6 | 156.6 | 147.6 |
| Diesel | 160.3 | 159.7 | 158.1 | 157.7 | 156.2 | 156.5 | 157.9 | 156.3 | 151.2 | 145.3 |
| AFV | 208.0 | 207.4 | 179.2 | 164.7 | 147.9 | 149.4 | 151.1 | 140.0 | 137.0 | 125.8 |

Table 2: Share of fuel type in new passenger cars (EU27)

| % | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Diesel | 31.0 | 35.9 | 40.7 | 44.4 | 47.9 | 49.1 | 50.3 | 51.9 | 51.3 | 45.1 |
| Petrol | 68.9 | 64.0 | 59.2 | 55.5 | 51.9 | 50.7 | 49.4 | 47.3 | 47.4 | 51.1 |
| Petrol-LPG | 0.00 | 0.01 | 0.04 | 0.01 | 0.02 | 0.08 | 0.23 | 0.59 | 1.14 | 3.61 |
| Other AFV | 0.11 | 0.10 | 0.09 | 0.10 | 0.14 | 0.17 | 0.10 | 0.14 | 0.16 | 0.14 |

Chart 1: Evolution of CO2 emissions from new passenger cars by fuel (EU27)

Table 3: Average CO2 emissions (gCO2/km) from new passenger cars by region

| EU aggregate | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EU27 |  |  |  |  |  |  |  | 158.7 | 153.6 | 145.7 |
| EU25 |  |  |  |  | 163.4 | 162.4 | 161.3 | 158.7 | 153.4 | 145.6 |
| EU15 | 172.2 | 169.7 | 167.2 | 165.5 | 163.7 | 162.6 | 161.5 | 158.8 | 153.3 | 145.2 |
| EU12 |  |  |  |  |  |  |  | 157.8 | 156.8 | 154.2 |
| EU10 |  |  |  |  | 157.2 | 158.1 | 157.3 | 157.7 | 155.5 | 153.0 |

The EU15 accounts for the vast majority of new passenger car registrations (94.6% in 2009). New cars in the EU15 are on average emitting 9 gCO2/km less than new cars in the EU12. While the EU15 average improved by 8.1 grams in 2009, the EU10 average improved by only 2.5 grams. The observation from previous monitoring reports, that new passenger cars registered in the EU15 have lower CO2 emissions than new passenger cars in the EU10, is re-confirmed in 2009. However, it is not clear at this stage if this can be attributed to a permanent behavioural change of citizens in the EU15, or if this is one of the effects of the financial and economic crisis which started in 2008. Data are also influenced by various scrappage schemes implemented in several countries in 2009.

Chart 2: Registrations of new passenger cars by region in year 2009

2.3.	Other car characteristics: engine power, engine capacity and mass

The average engine power of new diesel and petrol powered passenger cars had been increasing until 2007, it remained constant in 2008 and decreased in 2009. The average engine power of petrol powered vehicles decreased by 4 kW and offset the observed growth during last 7 years. Diesel powered vehicles decreased only by 2 kW to the levels of 2006. The 2009 data for the average engine power of AFV confirm the decreasing trend of previous years. The huge influence of petrol-LPG vehicles in Italy is the main driving factor for determining the average engine power of AFV.

Table 4: Average engine power of new passenger cars by fuel

| kW | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| All fuels | 77 | 79 | 80 | 81 | 83 | 84 | 84 | 80 |
| Petrol | 75 | 76 | 76 | 76 | 77 | 78 | 78 | 74 |
| Diesel | 81 | 82 | 83 | 85 | 89 | 91 | 91 | 89 |
| AFV | 77 | 74 | 69 | 69 | 69 | 61 | 61 | 58 |

The average engine capacity of new passenger cars in 2009 has seen a significant drop in 2009 by 83 cm3. Petrol powered vehicles observed the highest decrease (by 5% or by 77 cm3). The decrease in engine capacity of diesel powered vehicles is continuing and amounts to 37 cm3 in 2009 (-2%). The engine size difference between petrol and diesel powered vehicles was decreasing until 2005 when it reached 313 cm3 and since then it is widening again, and in 2009 the difference reached 378 cm3.

Table 5: Average engine capacity of new passenger cars by fuel

| cm3 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| All fuels | 1714 | 1731 | 1743 | 1730 | 1726 | 1724 | 1729 | 1703 | 1620 |
| Petrol | 1560 | 1570 | 1572 | 1571 | 1573 | 1561 | 1556 | 1531 | 1454 |
| Diesel | 1981 | 1961 | 1948 | 1904 | 1886 | 1885 | 1892 | 1869 | 1832 |
| AFV | 1602 | 1672 | 1628 | 1581 | 1561 | 1562 | 1424 | 1387 | 1328 |

The average mass of new passenger cars registered in the EU in 2009 decreased by 36 kg. The difference between petrol and diesel vehicles has been increasing slowly but constantly since 2004 (226 kg) and in 2009 it reached 292 kg. The average mass decreased by 10 kg for diesel vehicles in 2009. On the other hand, new petrol powered vehicles became on average lighter by 22 kg and alternative fuel vehicles by 68 kg. The observed decreases are mainly due to a consumer shift to smaller segments of vehicles (known as A and B segment) as a consequence of the financial/economic crisis and the design of the scrappage schemes in operation during 2009. Data before 2004 are not shown in the table due to problems with Member State reporting of mass during that period of time. For data before 2004 as well as detailed explanations of the origin of the mass data and all other mass related notes see Table 2 in the Annex to report COM(2009)9 final.

Table 6: Average mass of new passenger cars by fuel

| kg | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- |
| All fuels | 1347 | 1356 | 1372 | 1379 | 1373 | 1337 |
| Petrol | 1237 | 1235 | 1238 | 1235 | 1228 | 1206 |
| Diesel | 1463 | 1479 | 1501 | 1510 | 1508 | 1498 |
| Alter. fuel | 1415 | 1404 | 1392 | 1271 | 1237 | 1169 |

2.4.	Monitoring data by association

For reasons of continuity with previous reports, data in this section are presented per vehicle manufacturer association. In 2009, CO2 emissions from new passenger cars decreased for each association. In comparison to 2008, ACEA decreased its average emissions by 7.3 grams, JAMA by 11.2 grams, KAMA by 9.7 grams and other manufacturers on average by 18.4 grams. Both, KAMA and JAMA, had lower average emissions from new passenger cars in 2009 than ACEA while the situation was the reverse in 2007.

Table 7: Average CO2 emissions from new passenger cars by association

| gCO2/km | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACEA | 170.4 | 168.2 | 165.6 | 163.7 | 161.8 | 161.1 | 160.8 | 158.1 | 153.4 | 146.2 |
| JAMA | 180.9 | 177.9 | 174.9 | 173.3 | 170.9 | 167.4 | 162.5 | 160.6 | 154.7 | 143.6 |
| KAMA | 185.5 | 186.8 | 184.7 | 180.0 | 168.7 | 167.8 | 165.5 | 162.2 | 152.6 | 142.8 |
| Other | 195.1 | 190.8 | 185.6 | 180.2 | 182.3 | 187.6 | 188.5 | 200.0 | 190.1 | 171.8 |

Chart 3: Evolution of CO2 emissions from new passenger cars by association

The average mass of new passenger cars decreased for all manufacturers in 2009 between 36 and 46 kg. The average mass of a new passenger car from JAMA is 70 kg less than an average new car from ACEA. In the last 5 years, average emissions decreased by 10-16% for all associations while average mass changed between 0 and -2% (and vehicle power on average has not changed since 2004). This clearly indicates that specific CO2 emissions have been reduced without any significant downsizing of vehicle fleet.

Table 8: Average mass of new passenger cars by association

| kg | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACEA | 1 351 | 1 369 | 1 418 | 1 407 | 1 355 | 1 365 | 1 380 | 1 386 | 1 384 | 1 348 |
| JAMA | 1 308 | 1 379 | 1 375 | 1 380 | 1 310 | 1 309 | 1 322 | 1 335 | 1 315 | 1 278 |
| KAMA | 1 295 | 1 366 | 1 447 | 1 446 | 1 298 | 1 341 | 1 383 | 1 379 | 1 336 | 1 290 |
| Other | 1 363 | 1 288 | 1 302 | 1 279 | 1 264 | 1 258 | 1 309 | 1 383 | 1 291 | 1 251 |

The effect of the current financial crisis, economic downturn as well as the scrappage schemes is influencing the number of new passenger car registrations for each association during 2008 and 2009. When compared to 2007 (the highest number of registration in the last 10 years), there was a decrease of 7.8% in 2008 and of 9.9% in 2009. When comparing 2009 to 2008, there was an increase of 9.8% in registrations of KAMA while at the same time the registrations of ACEA and JAMA fell by 2.2% and 7.4% respectively. ACEA is still, by far, the major player on the European market, keeping its share at around 81% of all new registrations for the last 5 years.

Table 9: Registrations of new passenger cars by association (in thousands)

| '000 | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACEA | 10 864 | 11 088 | 11 603 | 11 472 | 12 164 | 12 027 | 12 121 | 12 473 | 11 599 | 11 344 |
| JAMA | 1 542 | 1 301 | 1 502 | 1 704 | 2 002 | 2 058 | 2 156 | 2 252 | 2 033 | 1 883 |
| KAMA | 415 | 322 | 325 | 437 | 630 | 736 | 711 | 754 | 644 | 707 |
| Other | 20 | 50 | 70 | 69 | 58 | 37 | 22 | 27 | 23 | 40 |
| Total | 12 840 | 12 761 | 13 500 | 13 682 | 14 853 | 14 858 | 15 010 | 15 505 | 14 299 | 13 975 |

3.	Statistical annex with data by Member State

Empty cells indicate that either data have not been submitted by the Member State concerned or, due to submitted data implausibility, it is not appropriate to show the value.

Table 10: Registration of new passenger cars by Member State [in thousands]

| '000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Austria | 295 | 280 | 300 | 311 | 308 | 309 | 298 | 294 | 319 |
| Belgium | 497 | 468 | 459 | 485 | 480 | 526 | 525 | 536 | 475 |
| Bulgaria |  |  |  |  |  |  | 86 | 91 | 21 |
| Cyprus |  |  |  | 20 | 18 | 20 | 25 | 24 | 16 |
| Czech Rep. |  |  |  | 115 | 105 | 107 | 126 | 134 | 159 |
| Denmark | 97 | 113 | 102 | 124 | 147 | 154 | 160 | 148 | 111 |
| Estonia |  |  |  | 17 | 20 | 25 | 31 | 24 | 10 |
| Finland | 106 | 113 | 145 | 141 | 146 | 143 | 123 | 137 | 89 |
| France | 2 228 | 2 120 | 1 988 | 1 996 | 2 059 | 1 986 | 2 050 | 2 037 | 2 259 |
| Germany | 3 342 | 3 122 | 3 237 | 3 267 | 3 319 | 3 445 | 3 126 | 3 067 | 3 786 |
| Greece | 245 | 242 | 203 | 264 | 274 | 279 | 294 | 279 | 221 |
| Hungary |  |  |  | 230 | 199 | 193 | 167 | 163 | 66 |
| Ireland | 117 | 152 | 146 | 154 | 171 | 177 | 186 | 151 | 56 |
| Italy | 2 430 | 2 278 | 2 244 | 2 264 | 2 237 | 2 325 | 2 494 | 2 163 | 2 160 |
| Latvia |  |  |  | 11 | 16 | 25 | 31 | 19 | 5 |
| Lithuania |  |  |  | 9 | 11 | 15 | 21 | 22 | 7 |
| Luxembourg | 22 | 44 | 44 | 48 | 49 | 51 | 51 | 52 | 47 |
| Malta |  |  |  | 4 | 7 | 6 | 6 | 5 | 6 |
| Netherlands | 526 | 507 | 487 | 479 | 452 | 478 | 494 | 493 | 396 |
| Poland |  |  |  | 297 | 230 | 223 | 264 | 305 | 221 |
| Portugal |  | 232 | 194 | 202 | 208 | 199 | 204 | 215 | 159 |
| Romania |  |  |  |  |  |  | 313 | 286 | 115 |
| Slovakia |  |  |  |  | 45 | 65 | 65 | 57 | 70 |
| Slovenia |  |  |  | 37 | 64 | 62 | 69 | 72 | 60 |
| Spain | 400 | 969 | 1 319 | 1 606 | 1 640 | 1 622 | 1 606 | 1 165 | 964 |
| Sweden | 223 | 249 | 257 | 260 | 269 | 278 | 300 | 248 | 209 |
| UK | 2 232 | 2 611 | 2 558 | 2 512 | 2 386 | 2 295 | 2 390 | 2 112 | 1 968 |

Table 11: Average mass of new passenger cars by Member State [in kg]

| kg | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Austria | 1 314 | 1 335 | 1 426 | 1 432 | 1 435 | 1 449 | 1 445 | 1 431 | 1 385 |
| Belgium | 1 288 | 1 319 | 1 361 | 1 375 | 1 396 | 1 407 | 1 423 | 1 425 | 1 406 |
| Bulgaria |  |  |  |  |  |  |  |  |  |
| Cyprus |  |  |  | 1 205 | 1 277 | 1 316 | 1 354 | 1 372 | 1 367 |
| Czech Rep. |  |  |  | 1 704 | 1 242 | 1 247 | 1 261 | 1 275 | 1 335 |
| Denmark |  | 1 306 | 1 325 | 1 327 | 1 324 | 1 328 | 1 370 | 1 320 | 1 313 |
| Estonia |  |  |  | 1 349 | 1 408 | 1 433 | 1 465 | 1 456 | 1 471 |
| Finland | 1 752 | 1 759 | 1 336 | 1 355 | 1 381 | 1 401 | 1 437 | 1 442 | 1 447 |
| France | 1 254 | 1 280 | 1 305 | 1 327 | 1 341 | 1 349 | 1 375 | 1 387 | 1 326 |
| Germany | 1 332 | 1 352 | 1 381 | 1 408 | 1 412 | 1 424 | 1 433 | 1 425 | 1 347 |
| Greece | 1 172 | 1 223 | 1 262 | 1 277 | 1 287 | 1 304 | 1 314 | 1 311 | 1 423 |
| Hungary |  |  |  | 1 182 | 1 203 | 1 237 | 1 264 | 1 288 | 1 330 |
| Ireland | 1 248 | 1 276 | 1 265 | 1 314 | 1 341 | 1 372 | 1 441 | 1 440 | 1 440 |
| Italy | 1 604 | 1 632 | 1 649 | 1 259 | 1 277 | 1 294 | 1 287 | 1 285 | 1 255 |
| Latvia |  |  |  | 1 452 | 1 445 | 1 468 | 1 502 | 1 498 | 1 535 |
| Lithuania |  |  |  | 1 433 | 1 448 | 1 483 | 1 481 | 1 467 | 1 486 |
| Luxembourg | 1 834 | 1 851 | 1 442 | 1 471 | 1 487 | 1 504 | 1 498 | 1 490 | 1 462 |
| Malta |  |  |  |  |  |  |  | 1 317 | 1 182 |
| Netherlands | 1 260 | 1 264 | 1 301 | 1 314 | 1 337 | 1 332 | 1 350 | 1 324 | 1 295 |
| Poland |  |  |  | 1 181 | 1 242 | 1 271 | 1 304 | 1 260 | 1 261 |
| Portugal |  | 1 229 | 1 254 | 1 295 | 1 329 | 1 352 | 1 365 | 1 352 | 1 344 |
| Romania |  |  |  |  |  |  | 1 268 | 1 286 | 1 291 |
| Slovakia |  |  |  |  | 1 174 |  |  |  |  |
| Slovenia |  |  |  | 1 246 | 1 305 | 1 316 | 1 340 | 1 350 | 1 346 |
| Spain | 1 266 | 1 725 | 1 317 | 1 335 | 1 374 | 1 395 | 1 416 | 1 400 | 1 394 |
| Sweden | 1 448 | 1 454 | 1 472 | 1 467 | 1 470 | 1 488 | 1 503 | 1 488 | 1 490 |
| UK | 1 347 | 1 356 | 1 392 | 1 387 | 1 374 | 1 390 | 1 394 | 1 380 | 1 358 |

Table 12: Average CO2 emissions from new passenger cars by Member State

| gCO2/km | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 | 2008 | 2009 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Austria | 165.6 | 164.4 | 163.8 | 161.9 | 162.1 | 163.7 | 162.9 | 158.1 | 150.2 |
| Belgium | 163.7 | 161.1 | 158.1 | 156.5 | 155.2 | 153.9 | 152.8 | 147.8 | 142.1 |
| Bulgaria |  |  |  |  |  |  | 171.6 | 171.5 | 172.1 |
| Cyprus |  |  |  | 173.4 | 173.0 | 170.1 | 170.3 | 165.6 | 160.7 |
| Czech Rep. |  |  |  | 154.0 | 155.3 | 154.2 | 154.2 | 154.4 | 155.5 |
| Denmark | 172.9 | 170.0 | 169.0 | 165.9 | 163.7 | 162.5 | 159.8 | 146.4 | 139.1 |
| Estonia |  |  |  | 179.0 | 183.7 | 182.7 | 181.6 | 177.4 | 170.3 |
| Finland | 178.1 | 177.2 | 178.3 | 179.8 | 179.5 | 179.2 | 177.3 | 162.9 | 157.0 |
| France | 159.8 | 156.8 | 155.0 | 153.1 | 152.3 | 149.9 | 149.4 | 140.1 | 133.5 |
| Germany | 179.5 | 177.4 | 175.9 | 174.9 | 173.4 | 172.5 | 169.5 | 164.8 | 154.0 |
| Greece | 166.5 | 167.8 | 168.9 | 168.8 | 167.4 | 166.5 | 165.3 | 160.8 | 157.4 |
| Hungary |  |  |  | 158.5 | 156.3 | 154.6 | 155.0 | 153.4 | 153.4 |
| Ireland | 166.6 | 164.3 | 166.7 | 167.6 | 166.8 | 166.3 | 161.6 | 156.8 | 144.4 |
| Italy | 158.3 | 156.6 | 152.9 | 150.0 | 149.5 | 149.2 | 146.5 | 144.7 | 136.3 |
| Latvia |  |  |  | 192.4 | 187.2 | 183.1 | 183.5 | 180.6 | 176.9 |
| Lithuania |  |  |  | 187.5 | 186.3 | 163.4 | 176.5 | 170.1 | 166.0 |
| Luxembourg | 177.0 | 173.8 | 173.5 | 169.7 | 168.6 | 168.2 | 165.8 | 159.5 | 152.5 |
| Malta |  |  |  | 148.8 | 150.5 | 145.9 | 147.8 | 146.9 | 135.7 |
| Netherlands | 174.0 | 172.4 | 173.5 | 171.0 | 169.9 | 166.7 | 164.8 | 156.7 | 146.9 |
| Poland |  |  |  | 154.1 | 155.2 | 155.9 | 153.7 | 153.1 | 151.6 |
| Portugal |  | 154.0 | 149.9 | 147.1 | 144.9 | 145.0 | 144.2 | 138.2 | 133.8 |
| Romania |  |  |  |  |  |  | 154.8 | 156.0 | 157.0 |
| Slovakia |  |  |  |  | 157.4 | 152.0 | 152.7 | 150.4 | 146.6 |
| Slovenia |  |  |  | 152.7 | 157.2 | 155.3 | 156.3 | 155.9 | 152.0 |
| Spain | 156.8 | 156.4 | 157.0 | 155.3 | 155.3 | 155.6 | 153.2 | 148.2 | 142.2 |
| Sweden | 200.2 | 198.2 | 198.5 | 197.2 | 193.8 | 188.6 | 181.4 | 173.9 | 164.5 |
| UK | 177.9 | 174.8 | 172.7 | 171.4 | 169.7 | 167.7 | 164.7 | 158.2 | 149.7 |

4.	Statistical annex with data by manufacturer

In the following table, the top 20 manufacturers based on the number of registrations in 2009 are shown. The complete database with data for all manufacturers is available on the Commission's website. The table includes a column labelled "Target" corresponding to the specific emissions target calculated in accordance with the formula set out in Annex I to Regulation (EC) No 443/2009. It is, however, important to note that in meeting the target manufacturers may take account of the flexibilities set out in Regulation (EC) No 443/2009: super-credits (Article 5), biofuels (Article 6), pooling (Article 7), derogations (Article 11), and eco-innovations (Article 12).

Table 13: Selected data by manufacturer

|  | Mass [kg] | Target gCO2/km | Average in 2009 gCO2/km | Registrations in 2009 |
| --- | --- | --- | --- | --- |
| VOLKSWAGEN | 1 396 | 131 | 150.7 | 1 595 889 |
| FORD | 1 268 | 125 | 139.7 | 1 249 195 |
| FIAT | 1 134 | 119 | 130.1 | 1 206 362 |
| RENAULT | 1 308 | 127 | 137.7 | 1 079 984 |
| PEUGEOT | 1 292 | 126 | 133.6 | 966 013 |
| CITROEN | 1 328 | 128 | 137.9 | 850 753 |
| OPEL | 1 329 | 128 | 147.6 | 802 686 |
| TOYOTA | 1 277 | 126 | 131.7 | 691 518 |
| BMW | 1 526 | 137 | 151.2 | 680 768 |
| DAIMLER | 1 487 | 135 | 166.8 | 666 341 |
| AUDI | 1 605 | 141 | 160.2 | 586 764 |
| SKODA | 1 266 | 125 | 147.9 | 446 671 |
| NISSAN | 1 348 | 129 | 154.3 | 359 094 |
| HYUNDAI | 1 245 | 124 | 137.8 | 322 432 |
| SEAT | 1 253 | 125 | 140.1 | 307 811 |
| KIA | 1 384 | 131 | 145.9 | 240 986 |
| VAUXHALL | 1 321 | 128 | 151.9 | 233 400 |
| HONDA | 1 354 | 129 | 147.3 | 231 810 |
| DACIA | 1 182 | 121 | 151.9 | 226 618 |
| MAZDA | 1 251 | 124 | 149.5 | 199 299 |

Of all manufacturers that registered more than 10 000 vehicles in the European Union in 2009, one manufacturer already managed to fully meet the target set for year 2015:
MARUTI (average mass: 932 kg, specific emissions target: 110 gCO2/km, average specific emissions in 2009: 104 gCO2/km, registrations in 2009: 14 095).

Decision (EC) 1753/2000 of the European Parliament and of the Council establishing a scheme to monitor the average specific emissions of CO2 from new passenger cars, OJ L 202, 10.8.2000.

Regulation (EC) 443/2009 of the European Parliament and of the Council of 23 April 2009 setting emission performance standards for new passenger cars, OJ L 140, 5.6.2009

More information related to the 0.7% correction can be found in COM(2002) 693 final and COM(2004) 78 final.

EU15 includes Austria, Belgium, Denmark, Finland, France, Germany, Greece, Ireland, Italy, Luxembourg, Netherlands, Portugal, Spain, Sweden and UK (excluding Northern Ireland).

EU10 includes Cyprus, Czech Republic, Estonia, Hungary, Latvia, Lithuania, Malta, Poland, Slovakia and Slovenia.

EU12 includes EU10, Bulgaria and Romania.

EU25 includes EU15 and EU10.

EU27 includes EU15 and EU12.

"Other" includes non-identified new passenger cars (vehicles for which Members States did not report correctly the required information)

Total registration of new passenger cars as covered by reporting EU Member States

http://ec.europa.eu/environment/air/transport/co2/co2_monitoring.htm

Specific emissions target based on formulae in Annex I of Regulation (EC) 443/2009